ARCHIMEDES

FEniCSx Edition

Gesture-driven post-processing for imported CAD models

What this builds

A solver-agnostic 3D post-processor: import geometry from Fusion, SolidWorks or Revit, mesh it, solve it with FEniCSx, then explore the result field with your hands — clip planes, isosurfaces, thresholds, probes, camera orbit.

The viewer consumes VTK and XDMF, which means it also opens results from Abaqus, Ansys, COMSOL, CalculiX and OpenFOAM. FEniCSx is your own solver, not a dependency of the viewer.

Five projects. Each one produces something that works on its own. The gesture layer is the last thing added, not the thing everything hangs off.

# Architecture

┌── INGEST ─────────────────────────────────────────────────┐ │ Fusion / SolidWorks → STEP, IGES │ │ Revit / ArchiCAD → IFC │ │ Any mesher → MSH, STL │ └───────────────────────────┬───────────────────────────────┘ ▼ ┌── MESH ───────────────────────────────────────────────────┐ │ Gmsh (OpenCASCADE kernel reads STEP natively) │ │ ifcopenshell → tessellation → Gmsh (BIM path) │ │ Physical groups tag faces for boundary conditions │ └───────────────────────────┬───────────────────────────────┘ ▼ ┌── SOLVE ──────────────────────────────────────────────────┐ │ DOLFINx: you write the weak form │ │ Linear elasticity · thermal · modal · coupled │ │ PETSc backend, MPI-parallel │ └───────────────────────────┬───────────────────────────────┘ ▼ ┌── RESULT FILES ───────────────────────────────────────────┐ │ XDMF + HDF5 (mesh + fields, time series) │ │ VTX / ADIOS2 (fast, native to DOLFINx) │ │ VTU (universal, what everyone else writes) │ └───────────────────────────┬───────────────────────────────┘ ▼ ┌── VIEWER (the actual product) ───────────────────────────┐ │ PyVista / VTK unstructured grid │ │ Filters: clip · threshold · contour · warp · probe │ │ Qt shell via pyvistaqt │ └───────────────────────────┬───────────────────────────────┘ ▲ ┌── GESTURE LAYER ──────────┴───────────────────────────────┐ │ MediaPipe hands in a QThread → Qt signals → filter calls │ └───────────────────────────────────────────────────────────┘

## Project map

| # | Project | Deliverable |
| --- | --- | --- |
| Project 1 | FEniCSx foundations | Install, weak forms, linear elasticity cantilever, von Mises, validated against theory |
| Project 2 | CAD import pipeline | STEP and IFC → Gmsh → DOLFINx → result file, with physical-group BC tagging |
| Project 3 | Viewer, mouse-driven | PyVista + Qt app: clip, threshold, contour, warp, probe, field switching |
| Project 4 | Gesture layer | MediaPipe QThread mapped onto the Project 3 controls |
| Project 5 | Depth and reach | Time scrubbing, modal animation, multi-solver ingest, spatial/AR mode |

PROJECT 1 · 3–4 weeks FEniCSx Foundations Write the weak form yourself, solve a cantilever, validate it

Milestone: python cantilever.py produces results.xdmf; tip deflection within 3% of PL³/3EI; ParaView shows the von Mises field.

## Install — do not fight this

DOLFINx is not cleanly pip-installable. Two options that actually work:

# Option A — Docker (most reliable, use this first) docker run -ti -v $(pwd):/root/shared -w /root/shared \ --shm-size=512m dolfinx/dolfinx:stable # Option B — conda (better for day-to-day dev, Linux/macOS) conda create -n fenicsx -c conda-forge \ fenics-dolfinx mpich pyvista python=3.12 conda activate fenicsx # Verify python -c "import dolfinx; print(dolfinx.__version__)" # Additional packages you will need across all five projects pip install gmsh meshio ifcopenshell pyvistaqt PySide6 \ mediapipe opencv-python

On Windows, use WSL2 with the conda route, or Docker Desktop. Native Windows DOLFINx is not worth the time.

## What you must understand before writing code

FEniCSx will not give you a beam element. You give it a variational statement of the problem and it discretises and solves it. That is the whole point — and it means you need three concepts:

| Concept | What it means |
| --- | --- |
| Weak form | The PDE multiplied by a test function and integrated by parts. Strong form: −∇·σ = f. Weak form: ∫ σ(u):ε(v) dx = ∫ f·v dx + ∫ T·v ds for all test functions v. |
| Function space | Where the solution lives. For 3D elasticity: continuous piecewise polynomials, vector-valued. In code: ('Lagrange', 1, (3,)) means linear Lagrange elements with 3 components. |
| Trial and test functions | The unknown u is the trial function; v is the arbitrary test function. The bilinear form a(u,v) becomes the stiffness matrix; the linear form L(v) becomes the load vector. |

Read the DOLFINx tutorial by Jørgen Dokken, chapters 1 through 3, before writing anything. The older FEniCS tutorials use a different API and will actively mislead you.

## Step-by-step walkthrough

Step 1 Build the mesh and function space Start with a built-in box mesh so you are debugging physics, not geometry. CAD import comes in Project 2.

# cantilever.py from mpi4py import MPI import numpy as np import ufl from dolfinx import mesh, fem, io, default_scalar_type from dolfinx.fem.petsc import LinearProblem # Geometry in metres L, Wd, H = 1.0, 0.05, 0.1 # length, width, height domain = mesh.create_box( MPI.COMM_WORLD, [np.array([0.0, 0.0, 0.0]), np.array([L, Wd, H])], [40, 4, 8], cell_type=mesh.CellType.hexahedron, ) # Vector-valued Lagrange space, degree 1, 3 components V = fem.functionspace(domain, ('Lagrange', 1, (domain.geometry.dim,)))

Step 2 Define the material and the constitutive relation Convert E and nu into the Lamé parameters, then write the strain and stress operators as plain Python functions of UFL expressions.

E = 210e9 # Pa nu = 0.3 rho = 7850.0 # kg/m3 g = 9.81 mu_ = E / (2.0 * (1.0 + nu)) lambda_ = E * nu / ((1.0 + nu) * (1.0 - 2.0 * nu)) def epsilon(u): return ufl.sym(ufl.grad(u)) # small-strain tensor def sigma(u): return (lambda_ * ufl.nabla_div(u) * ufl.Identity(len(u)) + 2.0 * mu_ * epsilon(u)) # isotropic linear elastic

Step 3 Write the weak form This is the line that matters. Everything above is setup; this is the physics.

u = ufl.TrialFunction(V) v = ufl.TestFunction(V) # Body force (self-weight) and surface traction f = fem.Constant(domain, default_scalar_type((0.0, 0.0, -rho * g))) T = fem.Constant(domain, default_scalar_type((0.0, 0.0, 0.0))) ds = ufl.Measure('ds', domain=domain) a = ufl.inner(sigma(u), epsilon(v)) * ufl.dx L_form = ufl.dot(f, v) * ufl.dx + ufl.dot(T, v) * ds

Step 4 Apply the clamped boundary condition Locate the facets at x = 0 geometrically, find their degrees of freedom, and pin all three components to zero.

def clamped(x): return np.isclose(x[0], 0.0) fdim = domain.topology.dim - 1 facets = mesh.locate_entities_boundary(domain, fdim, clamped) u_D = np.array([0.0, 0.0, 0.0], dtype=default_scalar_type) bc = fem.dirichletbc(u_D, fem.locate_dofs_topological(V, fdim, facets), V)

Step 5 Solve LinearProblem assembles, applies BCs, and hands off to PETSc. Use a direct LU solve while the problem is small.

problem = LinearProblem( a, L_form, bcs=[bc], petsc_options={'ksp_type': 'preonly', 'pc_type': 'lu'}, ) uh = problem.solve() uh.name = 'displacement'

Step 6 Post-process von Mises stress Stress is a derived quantity. Build the deviatoric part, form the von Mises scalar, and interpolate it into a discontinuous space (stress is discontinuous across elements for linear displacement elements).

s = sigma(uh) - (1.0 / 3.0) * ufl.tr(sigma(uh)) * ufl.Identity(len(uh)) von_mises = ufl.sqrt(1.5 * ufl.inner(s, s)) V_vm = fem.functionspace(domain, ('DG', 0)) expr = fem.Expression(von_mises, V_vm.element.interpolation_points()) vm = fem.Function(V_vm) vm.name = 'von_mises' vm.interpolate(expr)

Step 7 Write the result file XDMF plus HDF5 is the format to standardise on: it holds the mesh and multiple fields, supports time series, and every viewer reads it.

with io.XDMFFile(domain.comm, 'results.xdmf', 'w') as xdmf: xdmf.write_mesh(domain) xdmf.write_function(uh) xdmf.write_function(vm) # Quick sanity print tip = uh.x.array.reshape(-1, 3)[:, 2].min() print(f'Max downward tip deflection: {abs(tip)*1000:.3f} mm')

Step 8 Validate against theory Replace self-weight with a tip point load and check against the closed form. This is the step that tells you whether you have understood the formulation or just copied it.

# Analytical cantilever with tip load P: delta = P L^3 / (3 E I) I_yy = Wd * H**3 / 12.0 P = 1000.0 delta = P * L**3 / (3.0 * E * I_yy) print(f'Analytical: {delta*1000:.3f} mm') # Apply P as a traction over the end face: # traction magnitude = P / (Wd * H), applied on the x = L facets # Expect agreement within ~3% for a slender beam. # If the beam is stocky (L/H < 10) shear deformation makes FEM # legitimately softer than Euler-Bernoulli — that is not a bug.

Completion checklist — all must pass before the next project

☐ DOLFINx imports and prints a version number

☐ cantilever.py runs to completion with no PETSc errors

☐ Tip deflection under a tip load is within 3% of PL³/3EI for L/H ≥ 10

☐ results.xdmf opens in ParaView showing both displacement and von_mises

☐ Refining the mesh from [40,4,8] to [80,8,16] changes the answer by under 1% (convergence)

☐ You can explain, out loud, what the test function v is doing in the weak form

PROJECT 2 · 3–4 weeks CAD Import Pipeline Real geometry from Fusion, SolidWorks and Revit into a solved result

Milestone: Export a bracket from Fusion as STEP, run one command, get results.xdmf with the load applied to the face you tagged.

## The import routes

| Source | Export | Route |
| --- | --- | --- |
| Source | Export as | Path into the mesher |
| Fusion 360 | STEP (AP214) or IGES | Gmsh OCC kernel reads STEP directly — no conversion |
| SolidWorks | STEP AP214 or Parasolid x_t | STEP is the reliable route; Parasolid needs a licensed kernel |
| Onshape / Inventor / CATIA | STEP | Same as above |
| Revit / ArchiCAD | IFC | ifcopenshell tessellates to triangles, then Gmsh remeshes |
| Blender / mesh tools | STL, OBJ | Surface mesh only — needs volume meshing and cleanup |
| Existing FE meshes | MSH, INP, MED | meshio converts; skip Gmsh entirely |

## Step-by-step walkthrough

Step 1 Import STEP into Gmsh and mesh it Gmsh embeds the OpenCASCADE kernel, so it reads STEP and IGES natively. You do not need pythonOCC for this.

# meshing/step_to_msh.py import gmsh def mesh_step(step_path, msh_path, size_max=5.0, size_min=1.0, order=1): gmsh.initialize() gmsh.option.setNumber('General.Terminal', 1) gmsh.model.occ.importShapes(step_path) gmsh.model.occ.synchronize() # MUST call before meshing gmsh.option.setNumber('Mesh.CharacteristicLengthMax', size_max) gmsh.option.setNumber('Mesh.CharacteristicLengthMin', size_min) gmsh.option.setNumber('Mesh.ElementOrder', order) gmsh.option.setNumber('Mesh.Algorithm3D', 10) # HXT, fast tets gmsh.model.mesh.generate(3) gmsh.write(msh_path) gmsh.finalize() if __name__ == '__main__': mesh_step('bracket.step', 'bracket.msh', size_max=4.0)

Step 2 Tag faces with physical groups — this is the critical step Without physical groups you have a mesh but no way to say 'clamp this face' or 'load that face'. Tagging is what makes an imported CAD part usable for analysis. Strategy: find surfaces by their bounding box or centre of mass, then assign an integer tag. Those tags come through into DOLFINx as facet markers.

def tag_faces(tol=1e-6): """Assign physical groups to surfaces by geometric location.""" surfaces = gmsh.model.occ.getEntities(dim=2) volumes = gmsh.model.occ.getEntities(dim=3) # Whole-model bounding box, used to identify extreme faces xmin, ymin, zmin, xmax, ymax, zmax = gmsh.model.occ.getBoundingBox(-1, -1) fixed_faces, load_faces = [], [] for dim, tag in surfaces: com = gmsh.model.occ.getCenterOfMass(dim, tag) if abs(com[0] - xmin) < 1.0: # faces at the low-x end fixed_faces.append(tag) elif abs(com[0] - xmax) < 1.0: # faces at the high-x end load_faces.append(tag) # Tag numbers are what you reference in DOLFINx gmsh.model.addPhysicalGroup(2, fixed_faces, tag=1, name='fixed') gmsh.model.addPhysicalGroup(2, load_faces, tag=2, name='load') gmsh.model.addPhysicalGroup(3, [v[1] for v in volumes], tag=3, name='body')

Step 3 Read the tagged mesh into DOLFINx gmshio returns the mesh plus cell and facet tag objects. The facet tags carry your physical group numbers.

from dolfinx.io import gmshio from mpi4py import MPI domain, cell_tags, facet_tags = gmshio.read_from_msh( 'bracket.msh', MPI.COMM_WORLD, rank=0, gdim=3 ) # Sanity: what tags actually came through? print('facet tag values:', set(facet_tags.values)) # Expect {1, 2} matching the physical groups you created

Step 4 Apply BCs and loads using the tags, not coordinates This is the payoff. You now reference geometry by name rather than by hard-coded coordinates, so the same solver script works on any tagged part.

FIXED_TAG, LOAD_TAG = 1, 2 V = fem.functionspace(domain, ('Lagrange', 1, (3,))) fdim = domain.topology.dim - 1 # Clamp everything on the 'fixed' group fixed_facets = facet_tags.find(FIXED_TAG) bc = fem.dirichletbc( np.zeros(3, dtype=default_scalar_type), fem.locate_dofs_topological(V, fdim, fixed_facets), V, ) # Traction only on the 'load' group — note the subdomain_data argument ds = ufl.Measure('ds', domain=domain, subdomain_data=facet_tags) traction = fem.Constant(domain, default_scalar_type((0.0, 0.0, -1.0e6))) L_form = (ufl.dot(f, v) * ufl.dx + ufl.dot(traction, v) * ds(LOAD_TAG)) # ← tagged surface only

Step 5 The IFC route for Revit models IFC carries BIM semantics, not clean B-rep solids. ifcopenshell tessellates each element into triangles; you then either remesh the surface into a volume or work with the tessellation directly. Be realistic: BIM geometry is frequently dirty — non-manifold edges, self-intersections, missing caps. Expect cleanup work.

# meshing/ifc_to_mesh.py import ifcopenshell import ifcopenshell.geom import numpy as np def extract_elements(ifc_path, ifc_type='IfcBeam'): model = ifcopenshell.open(ifc_path) settings = ifcopenshell.geom.settings() settings.set(settings.USE_WORLD_COORDS, True) out = [] for el in model.by_type(ifc_type): try: shape = ifcopenshell.geom.create_shape(settings, el) except RuntimeError: continue # some elements have no geometry verts = np.array(shape.geometry.verts).reshape(-1, 3) faces = np.array(shape.geometry.faces).reshape(-1, 3) out.append({ 'name': el.Name, 'guid': el.GlobalId, 'verts': verts, 'faces': faces, }) return out # Then: write each element as STL, import into Gmsh, # and use gmsh.model.mesh.createGeometry() + createTopology() # to rebuild a volume from the surface triangulation.

Step 6 Wrap it into one command Build a single entry point so the whole pipeline is reproducible and scriptable. This is what Project 3 will call.

# pipeline.py import argparse from meshing.step_to_msh import mesh_step from solvers.elasticity import solve_elasticity def main(): ap = argparse.ArgumentParser() ap.add_argument('cad') # bracket.step ap.add_argument('--size', type=float, default=4.0) ap.add_argument('--E', type=float, default=210e9) ap.add_argument('--nu', type=float, default=0.3) ap.add_argument('--load', type=float, default=1.0e6) ap.add_argument('--out', default='results.xdmf') args = ap.parse_args() msh = args.cad.rsplit('.', 1)[0] + '.msh' mesh_step(args.cad, msh, size_max=args.size) solve_elasticity(msh, args.E, args.nu, args.load, args.out) # Usage: python pipeline.py bracket.step --size 3 --load 2e6

Problems you will actually hit

Mesh generation fails on imported STEP — usually slivers or tiny faces from the CAD tool. Fix: gmsh.option.setNumber('Geometry.OCCFixSmallFaces', 1) and set a sensible CharacteristicLengthMin so Gmsh does not try to resolve micron features.

Physical group tags do not appear in DOLFINx — you forgot gmsh.model.occ.synchronize() before addPhysicalGroup, or you tagged before meshing. Order matters: import, synchronize, tag, mesh, write.

Units. STEP files from Fusion are usually millimetres; your material constants are in Pascals and metres. Decide on one unit system at the mesh stage and scale there, not in three different places later.

Solve is very slow on a real part. A 500k-element tet mesh with LU factorisation will exhaust memory. Switch to an iterative solver: ksp_type 'cg', pc_type 'gamg' — algebraic multigrid is the right preconditioner for elasticity.

Completion checklist — all must pass before the next project

☐ A STEP file exported from Fusion meshes without errors

☐ facet_tags.find(1) returns a non-empty array of facet indices

☐ The traction is applied only to the tagged face (check the deformed shape makes physical sense)

☐ One command takes a STEP file to results.xdmf end to end

☐ A 200k-element mesh solves in under two minutes with CG plus GAMG

☐ An IFC file yields at least one element's geometry as vertices and faces

PROJECT 3 · 4–5 weeks The Viewer — Mouse-Driven First PyVista and Qt: clip, threshold, contour, warp, probe, field switching

Milestone: A desktop app that opens any XDMF or VTU file and lets you explore the field with sliders and buttons. Useful on its own, with no gestures.

Why mouse controls first

Every gesture in Project 4 is a remapping of a control built here. If the clip plane cannot be moved by a slider, it cannot be moved by a hand either.

Building the controls first means Project 4 is input remapping — a two-week job — rather than building a visualisation system and an input system simultaneously.

It also means that if you stop after this project, you still have a working, genuinely useful tool.

## Step-by-step walkthrough

Step 1 Load results and inspect what fields exist PyVista reads XDMF, VTU, VTK, and via meshio almost anything else. Always start by listing the available arrays — field naming differs between solvers.

# viewer/loader.py import pyvista as pv def load_result(path): """Load a result file and return a pyvista dataset.""" data = pv.read(path) # XDMF and multiblock files return a MultiBlock; flatten it if isinstance(data, pv.MultiBlock): data = data.combine() print('point arrays:', list(data.point_data.keys())) print('cell arrays: ', list(data.cell_data.keys())) print('n_points:', data.n_points, ' n_cells:', data.n_cells) return data def available_scalars(data): """All scalar fields the user can colour by.""" names = [] for k, arr in data.point_data.items(): names.append(k if arr.ndim == 1 else f'{k} (magnitude)') for k in data.cell_data.keys(): names.append(k) return names

Step 2 Build the Qt application shell pyvistaqt embeds a full VTK render window inside a Qt widget. Left side is the 3D view; right side is the control dock. In Project 4 the webcam panel goes into that same dock.

# viewer/app.py from PySide6 import QtWidgets, QtCore from pyvistaqt import QtInteractor import pyvista as pv class ViewerWindow(QtWidgets.QMainWindow): def __init__(self): super().__init__() self.setWindowTitle('Archimedes Viewer') self.resize(1400, 900) central = QtWidgets.QWidget() layout = QtWidgets.QHBoxLayout(central) # 3D view self.plotter = QtInteractor(central) self.plotter.set_background('#0F172A') layout.addWidget(self.plotter.interactor, stretch=4) # Control dock self.controls = ControlPanel(self) layout.addWidget(self.controls, stretch=1) self.setCentralWidget(central) self.data = None # full dataset self.actor = None # main mesh actor self.state = ViewState() def open_file(self, path): from .loader import load_result self.data = load_result(path) self.controls.populate_fields(available_scalars(self.data)) self.refresh()

Step 3 Hold all view parameters in one state object Every control writes into this object and calls refresh(). Gestures in Project 4 will write into the exact same object. This single decision is what makes the gesture layer a two-week job instead of a rewrite.

# viewer/state.py from dataclasses import dataclass, field @dataclass class ViewState: scalar: str = 'von_mises' cmap: str = 'viridis' clim: tuple = None # None = auto range clip_enabled: bool = False clip_origin: tuple = (0.0, 0.0, 0.0) clip_normal: tuple = (1.0, 0.0, 0.0) threshold_on: bool = False threshold_lo: float = 0.0 threshold_hi: float = 1.0 contour_on: bool = False contour_vals: list = field(default_factory=list) warp_on: bool = False warp_factor: float = 1.0 warp_vector: str = 'displacement' show_edges: bool = False opacity: float = 1.0

Step 4 Clip planes — use GPU clipping, not filtering The obvious approach is mesh.clip(), which builds a new dataset every time the plane moves. On a 500k-element mesh that is far too slow for continuous dragging. The right approach is a VTK clipping plane attached to the mapper. The full mesh stays on the GPU and clipping happens per-frame in hardware. Moving the plane becomes free.

# viewer/filters.py import vtk class ClipController: """GPU-side clipping — no re-filtering when the plane moves.""" def __init__(self, actor): self.plane = vtk.vtkPlane() self.plane.SetOrigin(0.0, 0.0, 0.0) self.plane.SetNormal(1.0, 0.0, 0.0) self.mapper = actor.GetMapper() self.enabled = False def enable(self): if not self.enabled: self.mapper.AddClippingPlane(self.plane) self.enabled = True def disable(self): if self.enabled: self.mapper.RemoveClippingPlane(self.plane) self.enabled = False def set_origin(self, x, y, z): self.plane.SetOrigin(x, y, z) # cheap — just a uniform update def set_normal(self, nx, ny, nz): self.plane.SetNormal(nx, ny, nz) # In the window: # self.clip = ClipController(self.actor) # ...on slider move: # self.clip.set_origin(x, 0, 0); self.plotter.render()

Keep mesh.clip() for the one case where GPU clipping is not enough: when you want to see the capped cross-section face with field values on it rather than a hollow cut. For that, filter once when the plane stops moving, not on every frame.

Step 5 Threshold, contour and warp These do require re-filtering, but they are used discretely rather than dragged continuously. Rebuild the actor when they change.

def build_display_mesh(data, state): """Apply the non-GPU filters in the correct order.""" m = data # Warp first — everything downstream should see deformed coords if state.warp_on and state.warp_vector in m.point_data: m = m.warp_by_vector(state.warp_vector, factor=state.warp_factor) if state.threshold_on: m = m.threshold( value=(state.threshold_lo, state.threshold_hi), scalars=state.scalar, ) if state.contour_on and state.contour_vals: m = m.contour(isosurfaces=state.contour_vals, scalars=state.scalar) return m def refresh(self): if self.data is None: return display = build_display_mesh(self.data, self.state) if self.actor is not None: self.plotter.remove_actor(self.actor) self.actor = self.plotter.add_mesh( display, scalars=self.state.scalar, cmap=self.state.cmap, clim=self.state.clim, show_edges=self.state.show_edges, opacity=self.state.opacity, scalar_bar_args={'title': self.state.scalar, 'color': 'white'}, ) self.clip = ClipController(self.actor) if self.state.clip_enabled: self.clip.enable() self.clip.set_origin(*self.state.clip_origin) self.clip.set_normal(*self.state.clip_normal) self.plotter.render()

Step 6 Point probe — report the field value under a screen position Given a screen coordinate, pick the 3D point and read the field there. This becomes the gesture 'point at a spot and hold' interaction in Project 4.

def probe_at_screen(self, sx, sy): """Return (world_point, value) for a screen pixel, or None.""" picker = vtk.vtkPointPicker() picker.SetTolerance(0.005) ok = picker.Pick(sx, sy, 0, self.plotter.renderer) if not ok: return None pid = picker.GetPointId() if pid < 0: return None world = picker.GetPickPosition() arr = self.data.point_data.get(self.state.scalar) if arr is None or pid >= len(arr): return None return world, float(arr[pid]) def show_probe_label(self, world, value): self.plotter.add_point_labels( [world], [f'{value:.3g}'], name='probe', font_size=14, point_color='yellow', point_size=12, )

Step 7 Camera control as explicit methods Do not rely only on VTK's built-in mouse interactor. Expose orbit, zoom and pan as methods so that Project 4 can call them directly from gesture deltas.

def orbit(self, d_azimuth, d_elevation): cam = self.plotter.camera cam.azimuth(d_azimuth) cam.elevation(max(-89.0, min(89.0, d_elevation))) self.plotter.render() def zoom(self, factor): self.plotter.camera.zoom(factor) self.plotter.render() def pan(self, dx, dy): cam = self.plotter.camera pos, foc = list(cam.position), list(cam.focal_point) for i in range(3): pos[i] += dx; foc[i] += dy cam.position, cam.focal_point = pos, foc self.plotter.render() def reset_view(self): self.plotter.reset_camera() self.plotter.view_isometric()

Completion checklist — all must pass before the next project

☐ The app opens any XDMF produced in Projects 1 and 2

☐ The field dropdown lists every scalar in the file and switching recolours the mesh

☐ The clip slider sweeps the plane smoothly at 30+ fps on a 200k-element mesh

☐ Threshold, contour and warp each produce visibly correct geometry

☐ Clicking a point shows a label with the correct field value

☐ Orbit, zoom, pan and reset all work as method calls, not just mouse drags

☐ Every control writes to ViewState and nothing else holds view parameters

PROJECT 4 · 4–5 weeks The Gesture Layer MediaPipe in a background thread, mapped onto the Project 3 controls

Milestone: Hands-free exploration: orbit the model, sweep a clip plane through it, probe a hotspot and read the value, all without touching the mouse.

## Gesture vocabulary for post-processing

This is a different vocabulary from an authoring interface. Every gesture here is continuous and approximate — there is nothing that needs a precise numeric value, which is exactly why the modality fits.

| Gesture | Action | Mapped to |
| --- | --- | --- |
| Gesture | Action | Mapped to |
| Open palm, move | Orbit camera | orbit(d_az, d_el) from palm centre delta |
| Two hands, spread apart | Zoom in | zoom(1 + k·Δspan) |
| Two hands, move together | Pan | pan(dx, dy) from midpoint delta |
| Index point, drag | Sweep clip plane along its normal | clip.set_origin() from fingertip depth |
| Flat hand, tilt | Set clip plane normal | clip.set_normal() from palm normal vector |
| Pinch, drag vertically | Threshold lower bound | state.threshold_lo, then refresh() |
| Two-finger pinch, rotate | Warp scale factor | state.warp_factor |
| Point and hold 1.0 s | Probe the point under the finger | probe_at_screen() + label |
| Swipe left / right | Cycle scalar field | next / previous in available_scalars() |
| Fist | Reset view | reset_view() |
| Open palm shake | Clear all filters | reset ViewState to defaults |

Design rules that make this usable rather than annoying

Split by hand role. Non-dominant hand controls the camera; dominant hand controls filters. Without this split every camera movement drags the clip plane with it.

Require an explicit engage gesture. Nothing should respond until the user makes a deliberate 'engage' pose — otherwise scratching your nose reorients the model.

Use relative deltas, never absolute positions. Map the change in hand position to the change in camera angle. Absolute mapping means the model jumps whenever tracking is briefly lost.

Smooth everything. Raw landmarks jitter by 3 to 5 pixels per frame. An exponential moving average at alpha 0.2 costs about 150 ms of lag and removes essentially all of it.

Add a deadband. Ignore movements below a threshold so the view holds still when the user is holding still.

## Step-by-step walkthrough

Step 1 Run MediaPipe in a QThread and emit Qt signals VTK rendering must happen on the main thread. The CV loop runs in a worker thread and communicates only through Qt signals, which Qt delivers on the receiving object's thread automatically.

# gesture/worker.py from PySide6 import QtCore import cv2, numpy as np import mediapipe as mp class HandWorker(QtCore.QThread): camera_delta = QtCore.Signal(float, float) # d_azimuth, d_elevation zoom_delta = QtCore.Signal(float) clip_position = QtCore.Signal(float) # 0..1 along the axis probe_request = QtCore.Signal(float, float) # screen x, y mode_changed = QtCore.Signal(str) frame_ready = QtCore.Signal(np.ndarray) def __init__(self, camera_index=0): super().__init__() self._running = False self.camera_index = camera_index def run(self): hands = mp.solutions.hands.Hands( max_num_hands=2, min_detection_confidence=0.7, min_tracking_confidence=0.7, ) cap = cv2.VideoCapture(self.camera_index) self._running = True prev = {} while self._running and cap.isOpened(): ok, frame = cap.read() if not ok: break frame = cv2.flip(frame, 1) res = hands.process(cv2.cvtColor(frame, cv2.COLOR_BGR2RGB)) if res.multi_hand_landmarks: self._dispatch(res, prev) self.frame_ready.emit(frame) cap.release() hands.close() def stop(self): self._running = False self.wait()

Step 2 Classify pose and emit deltas Keep the classifier simple and rule-based. Landmark 0 is the wrist, 4 is the thumb tip, 8 is the index tip, 5 and 17 are the index and pinky knuckles.

WRIST, THUMB, INDEX, MIDDLE, RING, PINKY = 0, 4, 8, 12, 16, 20 INDEX_PIP, MIDDLE_PIP, RING_PIP, PINKY_PIP = 6, 10, 14, 18 INDEX_MCP, PINKY_MCP = 5, 17 def extended(lm, tip, pip): return lm[tip].y < lm[pip].y def classify(lm): i = extended(lm, INDEX, INDEX_PIP) m = extended(lm, MIDDLE, MIDDLE_PIP) r = extended(lm, RING, RING_PIP) p = extended(lm, PINKY, PINKY_PIP) pinch = np.hypot(lm[THUMB].x - lm[INDEX].x, lm[THUMB].y - lm[INDEX].y) if pinch < 0.05: return 'PINCH' if i and not (m or r or p): return 'POINT' if i and m and r and p: return 'OPEN_PALM' if not (i or m or r or p): return 'FIST' return 'UNKNOWN' def palm_centre(lm): return np.array([lm[WRIST].x, lm[WRIST].y])

Step 3 Smooth, deadband, and convert to deltas Never emit a raw landmark position. Smooth it, difference it against the previous frame, and drop anything below the deadband.

# gesture/smoothing.py import numpy as np class EMA: def __init__(self, alpha=0.2): self.alpha, self.value = alpha, None def update(self, v): v = np.asarray(v, dtype=float) self.value = v if self.value is None else ( self.alpha * v + (1.0 - self.alpha) * self.value) return self.value class DeltaTracker: """Smoothed frame-to-frame delta with a deadband.""" def __init__(self, alpha=0.2, deadband=0.004): self.ema = EMA(alpha) self.prev = None self.deadband = deadband def update(self, v): cur = self.ema.update(v) if self.prev is None: self.prev = cur return np.zeros_like(cur) d = cur - self.prev self.prev = cur d[np.abs(d) < self.deadband] = 0.0 return d def reset(self): self.prev = None # call on gesture change to avoid a jump

Step 4 Dispatch to the right control based on hand role and pose Left hand drives the camera, right hand drives filters. A gesture change resets the delta tracker so the view does not jump when the user switches pose.

def _dispatch(self, res, prev): for lms, handed in zip(res.multi_hand_landmarks, res.multi_handedness): lm = lms.landmark side = handed.classification[0].label # 'Left' or 'Right' pose = classify(lm) key = (side, pose) if prev.get(side) != pose: self.trackers[side].reset() # pose changed — no jump prev[side] = pose self.mode_changed.emit(f'{side}: {pose}') centre = palm_centre(lm) d = self.trackers[side].update(centre) # LEFT HAND — camera if side == 'Left' and pose == 'OPEN_PALM': self.camera_delta.emit(float(d[0] * 180.0), float(-d[1] * 180.0)) # RIGHT HAND — filters elif side == 'Right' and pose == 'POINT': self.clip_position.emit(float(lm[INDEX].x)) elif side == 'Right' and pose == 'FIST': self.mode_changed.emit('RESET')

Step 5 Connect the worker to the viewer This is the entire integration. Because Project 3 exposed everything as methods on ViewState and the window, wiring gestures is a handful of signal connections.

# In ViewerWindow.start_gestures() from gesture.worker import HandWorker def start_gestures(self): self.worker = HandWorker(camera_index=0) self.worker.camera_delta.connect(self.orbit) self.worker.zoom_delta.connect(self.zoom) self.worker.clip_position.connect(self.on_gesture_clip) self.worker.probe_request.connect(self.on_gesture_probe) self.worker.frame_ready.connect(self.hud.show_frame) self.worker.mode_changed.connect(self.hud.show_mode) self.worker.start() def on_gesture_clip(self, t): """t in 0..1 → position along the model's x extent.""" xmin, xmax = self.data.bounds[0], self.data.bounds[1] x = xmin + t * (xmax - xmin) self.state.clip_enabled = True self.state.clip_origin = (x, 0.0, 0.0) self.clip.enable() self.clip.set_origin(x, 0.0, 0.0) self.plotter.render() # cheap — GPU clipping, no refilter def closeEvent(self, event): if hasattr(self, 'worker'): self.worker.stop() # never leave the thread running event.accept()

Step 6 Add the HUD A small dock showing the webcam feed with landmarks drawn, the current pose per hand, and a dwell progress bar for the probe gesture. Without visible feedback the interface feels broken even when it works.

# gesture/hud.py from PySide6 import QtWidgets, QtGui, QtCore import cv2 class GestureHUD(QtWidgets.QWidget): def __init__(self): super().__init__() lay = QtWidgets.QVBoxLayout(self) self.lbl_mode = QtWidgets.QLabel('idle') self.lbl_mode.setStyleSheet('font-family:monospace;font-size:13px;') self.lbl_cam = QtWidgets.QLabel() self.lbl_cam.setFixedSize(280, 210) self.bar = QtWidgets.QProgressBar(); self.bar.setRange(0, 100) lay.addWidget(self.lbl_mode); lay.addWidget(self.lbl_cam) lay.addWidget(self.bar); lay.addStretch() def show_frame(self, bgr): rgb = cv2.cvtColor(bgr, cv2.COLOR_BGR2RGB) h, w, c = rgb.shape img = QtGui.QImage(rgb.data, w, h, w * c, QtGui.QImage.Format_RGB888) self.lbl_cam.setPixmap( QtGui.QPixmap.fromImage(img).scaled( self.lbl_cam.size(), QtCore.Qt.KeepAspectRatio, QtCore.Qt.SmoothTransformation)) def show_mode(self, text): self.lbl_mode.setText(text)

Completion checklist — all must pass before the next project

☐ MediaPipe runs at 25+ fps without dropping the viewer below 30 fps

☐ Left-hand open palm orbits smoothly with no visible jitter

☐ Right-hand point sweeps the clip plane continuously through the model

☐ Switching pose does not cause the camera or clip plane to jump

☐ Point-and-hold for one second probes and labels the correct value

☐ Fist resets the view; palm shake clears all filters

☐ Closing the window stops the thread cleanly, with no zombie process

☐ A two-minute recorded demo: orbit, clip sweep, field switch, probe — hands only

PROJECT 5 · 6–8 weeks Depth and Reach Time series, modal animation, multi-solver ingest, spatial mode

Milestone: The viewer opens an Abaqus or Ansys export as readily as your own; mode shapes animate; a load history scrubs with a hand sweep.

## 5A — Time and load step scrubbing

XDMF supports time series natively. Write each step with a time value, then scrub through them. A horizontal hand sweep maps to the step index — one of the most natural gesture mappings in the whole interface.

# Writing a time series from DOLFINx with io.XDMFFile(domain.comm, 'transient.xdmf', 'w') as xdmf: xdmf.write_mesh(domain) for step, t in enumerate(time_values): # ... solve for this step, update uh ... xdmf.write_function(uh, t) # Reading and scrubbing in the viewer reader = pv.get_reader('transient.xdmf') print('time values:', reader.time_values) def set_time_step(self, index): index = max(0, min(index, len(self.reader.time_values) - 1)) self.reader.set_active_time_point(index) self.data = self.reader.read().combine() self.refresh()

## 5B — Modal analysis and mode shape animation

Modal analysis is a generalised eigenvalue problem, K φ = ω² M φ. SLEPc ships with the DOLFINx conda package and solves it directly.

# solvers/modal.py from slepc4py import SLEPc from dolfinx.fem.petsc import assemble_matrix import numpy as np def solve_modes(V, a_form, m_form, bcs, n_modes=6): K = assemble_matrix(fem.form(a_form), bcs=bcs); K.assemble() M = assemble_matrix(fem.form(m_form), bcs=bcs); M.assemble() eps = SLEPc.EPS().create(V.mesh.comm) eps.setOperators(K, M) eps.setProblemType(SLEPc.EPS.ProblemType.GHEP) # generalised Hermitian eps.setWhichEigenpairs(SLEPc.EPS.Which.SMALLEST_MAGNITUDE) eps.setDimensions(nev=n_modes) eps.setFromOptions() eps.solve() modes = [] for i in range(eps.getConverged()): lam = eps.getEigenvalue(i).real vec = K.createVecRight() eps.getEigenvector(i, vec) freq = np.sqrt(abs(lam)) / (2.0 * np.pi) # Hz modes.append((freq, vec.array.copy())) return modes # Animation in the viewer: scale the mode shape by sin(2*pi*f*t) # and re-warp each frame with a QTimer at 30 fps.

## 5C — Multi-solver ingest — the feature that makes this general

Once the viewer reads other vendors' results, it stops being a companion to your solver and becomes a tool anyone can use. This is the highest-leverage work in Project 5.

| Solver | Export | Reader |
| --- | --- | --- |
| Source | Export route | Reader |
| Abaqus | Open the ODB, File → Export → VTK, or use odbAccess scripting | pv.read() on the VTU |
| Ansys Mechanical | Export result as VTK, or read RST with the pyansys / ansys-dpf toolchain | pv.read(), or ansys-dpf-core |
| COMSOL | Export → Data → VTU | pv.read() |
| CalculiX | FRD output → ccx2paraview → VTU | pv.read() |
| OpenFOAM | Native, or foamToVTK | pv.OpenFOAMReader |
| LS-DYNA | d3plot via lasso-python | convert to VTU with meshio |

# viewer/ingest.py — normalise field names across vendors FIELD_ALIASES = { 'von_mises': ['von_mises', 'S_Mises', 'SEQV', 'vonMises', 'sigma_vm', 'Stress:Equivalent (von-Mises)'], 'displacement':['displacement', 'U', 'Displacement', 'disp'], 'temperature': ['temperature', 'NT11', 'TEMP', 'T'], } def normalise_fields(dataset): """Rename vendor-specific arrays to canonical names.""" for canonical, aliases in FIELD_ALIASES.items(): for alias in aliases: if alias in dataset.point_data and alias != canonical: dataset.point_data[canonical] = dataset.point_data[alias] break if alias in dataset.cell_data and alias != canonical: dataset.cell_data[canonical] = dataset.cell_data[alias] break return dataset

## 5D — Spatial and AR mode

Two credible routes, depending on whether you want a browser or a headset.

| Route | Approach |
| --- | --- |
| Browser (WebXR) | Export the display mesh to glTF with vertex colours baked from the colour map, load it in Three.js with WebXR enabled. Works on any phone with a browser. Lowest effort, widest reach. |
| Headset (Unity or Unreal) | Export glTF or FBX, build a Quest or Vision Pro app. Native hand tracking is far better than webcam MediaPipe, and the interaction genuinely improves. Much more work, much better result. |
| Desktop depth camera | An Intel RealSense gives true 3D hand positions and removes the depth ambiguity that limits webcam gestures. A middle path that improves the existing app rather than replacing it. |

# Export the current view for AR def export_gltf(self, path): display = build_display_mesh(self.data, self.state) display = display.copy() # Bake the colour map into vertex colours scalars = display.point_data[self.state.scalar] lo, hi = (self.state.clim if self.state.clim else (scalars.min(), scalars.max())) import matplotlib.cm as cm norm = (scalars - lo) / max(hi - lo, 1e-12) rgba = (cm.get_cmap(self.state.cmap)(norm) * 255).astype('uint8') display.point_data['RGBA'] = rgba pl = pv.Plotter(off_screen=True) pl.add_mesh(display, scalars='RGBA', rgba=True) pl.export_gltf(path)

Completion checklist — all must pass before the next project

☐ A transient XDMF scrubs through all time steps with a hand sweep

☐ The first six natural frequencies match an analytical or commercial reference within 5%

☐ Mode shapes animate smoothly at 30 fps

☐ The viewer opens a VTU exported from at least one commercial solver

☐ Field name normalisation maps that solver's stress array to von_mises automatically

☐ glTF export opens in a browser WebXR viewer with correct colours

# Timeline and Study Plan

| Weeks | Work | Study |
| --- | --- | --- |
| Weeks | Project work | Study alongside |
| 1–2 | Install DOLFINx via Docker, run the tutorial demos unmodified | Dokken DOLFINx tutorial ch. 1–2; weak forms |
| 3–4 | Write cantilever.py from scratch, validate against PL³/3EI | Dokken ch. 3; linear elasticity theory |
| 5–6 | Gmsh STEP import, meshing options, mesh quality | Gmsh Python API tutorials t1–t20 |
| 7–8 | Physical group tagging, tagged BCs, one-command pipeline | Dokken ch. on subdomains and boundary conditions |
| 9–11 | PyVista basics, Qt shell, ViewState, field switching | PyVista examples gallery; PySide6 layouts |
| 12–13 | GPU clip planes, threshold, contour, warp | VTK mapper and clipping plane documentation |
| 14 | Point probe, camera methods, polish | VTK picking documentation |
| 15–16 | MediaPipe worker thread, classifier, smoothing | MediaPipe Hands guide; Qt threading docs |
| 17–18 | Signal wiring, HUD, tuning deadbands and gains | Nothing new — this is iteration and feel |
| 19–20 | Record the demo, write it up, publish the repo | — |
| 21–26 | Project 5 features, chosen by what you want to show | SLEPc docs; glTF and WebXR if going spatial |

## Resources that actually matter for this stack

| Resource | Why |
| --- | --- |
| DOLFINx tutorial — Jørgen Dokken | The only FEniCSx resource you need to start. Current API, well explained. Older FEniCS tutorials will mislead you — ignore them. |
| FEniCSx demo collection | Ships with the installation. The elasticity, hyperelasticity and eigenvalue demos are directly relevant. |
| Gmsh Python API tutorials (t1–t20) | In the Gmsh source tree. t16 onwards covers OCC and STEP import specifically. |
| PyVista examples gallery | Extremely good. Almost every filter you need has a runnable example. |
| VTK textbook and Doxygen docs | For when PyVista does not expose what you need — clipping planes, pickers, custom mappers. |
| MediaPipe Hands solution guide | Landmark indices, confidence tuning, chirality handling. |
| SOFA Framework | Real-time FEM used in soft robotics and surgical simulation. Worth studying if the robotics direction firms up. |
| Bathe — Finite Element Procedures | Free PDF from MIT. The reference for when you need to know why something behaves the way it does. |

The ordering discipline that makes this work

Project 3 before Project 4 is the load-bearing decision. Build every control as a method that writes to ViewState, and the gesture layer becomes signal wiring rather than a system rewrite.

Each project ends with something usable. After Project 2 you have a CAD-to-FEA pipeline. After Project 3 you have a viewer worth using. The gesture layer is added value, not the foundation.

If you stall, stall late. A finished Project 3 is a real contribution. A half-finished Project 1 is nothing.

Archimedes FEniCSx Edition  ·  Five projects  ·  CAD in, hands on the result